FINAL YEAR PROJECT MASTERBOOK

Local AI Evaluation for a
CAD-Aware Shock-Isolator Assistant

From local model sizing and tool calling to embedding selection, RAG, and the frozen 170-case ablation study

A reusable technical record for thesis writing, repository documentation, and presentation development

Progress edition

15 July 2026

CAD-Aware AI Assistant - Shock Isolator Selection

Problem Statement

Engineering calculations frequently require information to be transferred manually between CAD models, spreadsheets, manufacturer datasheets and reports. This fragmented workflow creates opportunities for transcription errors, unit mismatches, incomplete inputs and loss of provenance, making final safety factors and design verdicts difficult to audit.

This project proposes a locally deployed engineering agent in which responsibilities are separated according to their reliability requirements. The local LLM interprets requests and coordinates actions; deterministic Python tools perform engineering calculations and produce PASS/FAIL decisions; and local retrieval-augmented generation supplies relevant references and citations.

The research investigates how effectively this architecture mitigates the accuracy, safety and traceability risks associated with the fragmented workflow. A controlled component-ablation experiment compares a local LLM alone, the same LLM with deterministic engineering tools, and the same system with local RAG. The evaluation uses a frozen 170-case benchmark derived from the identified workflow failure modes, including numerical transfer, unit conversion, missing-input, adversarial-safety, multi-tool and reference-provenance cases.

The study evaluates the correctness, safety and traceability of the proposed integration mechanisms. Human productivity and usability effects are outside the present experimental scope and are reserved for future work.

Document status and intended use

Status. This is a living masterbook. It records completed exploratory studies, the frozen final protocol, current System B results, and explicit placeholders for work that is still running. It must not be presented as a completed final-results chapter until Systems C and D and the paired statistical analysis are finished.

This document is deliberately more complete than a normal thesis chapter. Its sections are written as reusable blocks so that material can later be shortened for the final report, copied into repository documentation, or converted into presentation slides without reconstructing the project history from raw logs.

| Use | Recommended extraction |
| --- | --- |
| FYP report | Use Sections 1-4 for background and methodology; use Sections 8-12 for experimental design and results. |
| Progress update | Use the executive summary, screening tables, freeze record, and current-results section. |
| Presentation | Use the architecture flow, two screening figures, B/C/D table, benchmark composition, and System B result table. |
| Repository | Use the evidence map and dataset-path section; link to raw JSONL instead of pasting it. |

Contents

| Part | Sections |
| --- | --- |
| Executive summary | Current decisions, experiment, and System B status |
| Part I - Foundations | 1-5: architecture, transformer/tool/RAG flow, VRAM, quantization, and serving |
| Part II - Local LLM screening | 6-9: selection criteria, controlled pilot, results, failure analysis, and decision |
| Part III - Embeddings and RAG | 10-13: retrieval role, corpus, candidate comparison, frozen BGE-M3 configuration |
| Part IV - Frozen evaluation | 14-21: hypotheses, B/C/D ablation, benchmark, dataset path, metrics, credibility, and hashes |
| Part V - Results and progress | 22-27: pilot checks, System B result, C/D placeholders, challenges, next work, and validity |
| Part VI - Learning and evidence | 28-30: learning sequence, repository evidence map, and slide extraction map |
| References and appendices | Sources, terminology, and report-ready method/selection paragraphs |

Executive summary

This project evaluates a locally hosted language model as the controller of a deterministic shock-isolator engineering system. The language model does not determine the physics result. It interprets the request, selects a Python tool, supplies arguments, and explains the returned result. Catalogue data, unit conversion, shock response, static-load checks, travel checks, and PASS/FAIL decisions remain under deterministic Python control.

The laptop has an NVIDIA RTX 5080 Laptop GPU with approximately 16 GB of VRAM. This constraint motivated Q4_K_M quantization and an 8,192-token context. Three local instruction models were screened on the same ten development cases. Qwen3.5 9B was selected because it was the only candidate to combine correct safety verdicts, correct tool route, correct missing-input behaviour, and fully parseable verdict envelopes in the exploratory pilot.

The project then replaced hosted embeddings with a local embedding layer. BGE-M3, Qwen3 Embedding 8B, and Mixedbread MXBAI Embed Large v1 were compared on the same shock-mount corpus and relevance judgements. BGE-M3 was selected because it achieved the strongest mean reciprocal rank and perfect Hit@3 in the development test. The final frozen BGE-M3 index contains 34 shock-mount reference chunks.

The final experiment is a controlled B/C/D component ablation on 170 frozen shock-mount cases, repeated three times per system. System B is the local model without tools or RAG. System C adds deterministic engineering tools. System D adds the frozen local RAG tool. The main safety metric is false-safe rate, supported by verdict accuracy, ASK handling, tool routing, arguments, retrieval, citations, consistency, latency, Wilson confidence intervals, and paired McNemar tests.

Current result. System B has completed all 510 runs. Per-run verdict accuracy is 162/420 (38.6%), false-safe rate is 12/87 (13.8%), parseable-envelope rate is 501/510 (98.2%), consistency is 129/140 cases (92.1%), and no hard runtime failures occurred. System C currently contains 66/510 raw records; System D has not started at this report snapshot.

Part I - Foundations and system architecture

1. Project purpose and two-layer validation

The project combines mechanical-engineering validation with agent evaluation. Keeping these layers separate prevents a good conversational result from being mistaken for proof that the underlying physics is correct.

| Layer | Authority | What is validated |
| --- | --- | --- |
| Engineering layer | Python catalogue and physics engine | Equations, units, catalogue values, load distribution, static gate, dynamic travel, transmitted acceleration, and final PASS/FAIL result. |
| Agent layer | Local LLM plus LangChain execution loop | Tool selection, argument extraction, missing-input policy, ordered calls, output envelope, explanation, and optional reference retrieval. |
| CAD layer | SolidWorks COM extraction | Mass, centre of gravity, and bounding-box inputs. The CAD integration is intentionally thin. |

Interpretation rule. A perfect agent score proves agreement with the frozen Python oracle. It does not independently certify the oracle or replace physical qualification testing.

2. What happens inside the local language model

A language model receives text as tokens. Each token is mapped to a vector embedding. Transformer layers repeatedly update those vectors using attention and feed-forward networks. The final vectors are mapped to probabilities for the next token. Generation repeats one token at a time until the answer or tool call is complete.

User text
  -> tokenizer converts text to token IDs
  -> token IDs become vectors
  -> transformer attention and feed-forward layers update the vectors
  -> output probabilities are produced
  -> the next text token or structured tool-call token is selected

Tool calling is not normally performed by embedding the user question and finding the nearest tool-description vector. In this project, the model receives the tool names, descriptions, and JSON argument schemas as part of its input. It then predicts a structured call, such as `select_isolator` with named arguments. LangChain executes the corresponding Python function and sends the tool result back to the model for the final response.

RAG is a separate retrieval process. The query is embedded, compared with stored document vectors, and the top matching passages are returned. In this project, RAG is exposed as the `lookup_knowledge` tool. It is therefore agentic and selective rather than an always-on preprocessing stage.

Question
  -> Qwen3.5 chooses a route
     -> numerical/design task: call deterministic engineering tool
     -> reference/explanation task: call lookup_knowledge
  -> Python tool or vector store returns evidence
  -> Qwen3.5 writes the answer and JSON verdict envelope

3. Local LLM memory and VRAM sizing

The practical local-model constraint is GPU memory. A useful first estimate is written in ordinary words rather than compressed mathematical notation:

VRAM estimate. Required GPU memory is approximately the model weights, plus the active KV cache, plus runtime overhead for CUDA and temporary activations.

Model weights are the largest fixed component for this project. A rough weight-only estimate is:

Approximate weight memory = number of parameters x quantization bits / 8
Example: 9.7 billion parameters x 4 bits / 8 is about 4.85 GB before format overhead.
The installed Qwen3.5 9B GGUF artifact is 6.59 GB because real files include metadata and quantization structures.

The KV cache is working memory for tokens already processed in the current context. Every autoregressive transformer uses a KV cache during efficient generation; it is not unique to quantized models. An 8K context means the model can hold up to roughly 8,192 active input and output tokens in the current request. It does not mean 8 KB or 8 GB.

RAG does not make the complete external database occupy the LLM context. The 34 reference chunks remain in the JSON vector index. Only the top three retrieved excerpts, each capped at 1,400 characters in the frozen configuration, are inserted into the live tool result. This is the relationship between external RAG storage and the 8K model context.

| Memory component | Behaviour | Project implication |
| --- | --- | --- |
| Quantized model weights | Fixed while the model is loaded | Q4_K_M makes 9B-14B candidates feasible on 16 GB VRAM. |
| KV cache | Grows with configured/active context | Keep the evaluation alias at 8,192 tokens; do not allocate the model's theoretical maximum context. |
| Runtime overhead | CUDA context and temporary tensors | Leave headroom instead of filling all 16 GB with weights. |
| Embedding model | Loaded only when an embedding request is served | BGE-M3 is much smaller than the LLM; sequential Ollama serving can unload/reload models if necessary. |
| RAG index | Stored mainly in system memory/disk | Only retrieved excerpts enter the LLM context. |

4. Why quantization was selected

Quantization stores model weights at lower precision. Q4_K_M is a four-bit family designed to preserve more important weights with mixed quantization while greatly reducing memory use. The trade-off is a possible loss of reasoning and argument fidelity compared with higher precision. It was used because full or half-precision 9B-14B weights leave insufficient room on the 16 GB GPU for context and runtime overhead.

Quantization compresses the model weights; it does not remove the KV cache.

All three screened LLM candidates used Q4_K_M, so quantization level was held constant during model selection.

A quantized inference artifact is normally produced after training. For future adaptation, the practical sequence is: train a LoRA/QLoRA adapter, merge it if required, quantize the resulting inference model, then serve it with Ollama or llama.cpp.

Fine-tuning is not part of the current B/C/D experiment. It remains a later extension if tool-call or argument failures justify it.

5. Local serving environment

| Item | Frozen or observed value |
| --- | --- |
| CPU | Intel Core Ultra 9 275HX |
| System memory | 31.4 GiB usable |
| GPU | NVIDIA GeForce RTX 5080 Laptop GPU |
| VRAM | 16,303 MiB, approximately 16 GB |
| Operating system | Windows; native Ollama serving |
| Python | 3.10.8; repository virtual environment at `mpd` |
| Ollama | 0.31.2; OpenAI-compatible endpoint `http://localhost:11434/v1` |
| Selected LLM alias | `cad-eval-qwen35-9b:8k` |
| Selected embedder | `bge-m3` through Ollama `/api/embed` |

Ollama may unload a model after its keep-alive period. An empty `ollama ps` therefore means the model is no longer resident in VRAM, not that the application secretly switched to a cloud endpoint. The evaluation runner sends an unscored warm-up request before timed cases.

Part II - Local language-model screening

6. Selection criteria and candidate pool

The correct local model is not simply the largest model that fits. The controller must reliably call tools, preserve explicit inputs, request missing required data, and produce an auditable output. The screening therefore prioritised safety behaviour and interface reliability over general-chat impressions.

| Criterion | Why it matters |
| --- | --- |
| VRAM fit | Weights, KV cache, and overhead must remain within the 16 GB GPU. |
| Tool routing | The correct deterministic Python function must be selected. |
| Argument extraction | Non-default masses, units, mount counts, objectives, pulse shapes, and limits must be preserved. |
| Missing-input behaviour | The model must return ASK rather than inventing a mass or other required value. |
| Verdict fidelity | The final PASS/FAIL/ASK result must agree with the tool output. |
| Envelope compliance | The response must contain parseable structured JSON for deterministic auditing. |
| Latency | The model must be practical for an interactive engineering assistant. |

| Candidate | Parameters | Stored Q4 artifact | Evaluation alias |
| --- | --- | --- | --- |
| Qwen3.5 9B | 9.7B | 6.59 GB | `cad-eval-qwen35-9b:8k` |
| Qwen3 14B | 14.8B | 9.28 GB | `cad-eval-qwen3-14b:8k` |
| Gemma 4 12B | 11.9B | 7.56 GB | `cad-eval-gemma4-12b:8k` |

7. Controlled exploratory screening method

Each model answered the same ten shock-mount development cases once. Tools were enabled and RAG was disabled. The same prompt, tool schemas, verdict contract, serving backend, hardware, and inference settings were used. One unscored warm-up turn preceded measurement.

| Controlled variable | Value |
| --- | --- |
| Quantization | Q4_K_M for every candidate |
| Context | 8,192 tokens |
| Temperature | 0.0 |
| Seed | 42 where supported |
| Maximum output | 2,048 tokens |
| Tools | Enabled |
| RAG | Disabled |
| Conversation history | None; independent single-turn cases |
| Scoring | Deterministic gold tools, arguments, verdicts, and envelope parsing; no LLM judge |

The ten cases covered default selection, explicit mount counts, the best-isolation objective, pound-to-kilogram conversion, passing and failing verification, half-sine pulse extraction, catalogue lookup, missing mass, and a two-tool workflow. The dataset is development evidence and was allowed to influence the prompt before the final freeze.

Development correction. The first Qwen3.5 missing-mass answer asked for mass in prose but omitted the required ASK JSON envelope. An explicit ASK example was added to the evaluation-only prompt. The relevant two cases then improved from 1/2 to 2/2 verdict accuracy and from 1/2 to 2/2 parseable envelopes. Production/UI prompting and engineering tools were not changed.

8. Local LLM screening results

| Metric | Qwen3.5 9B | Qwen3 14B | Gemma 4 12B |
| --- | --- | --- | --- |
| Safety verdict accuracy | 9/9 (100.0%) | 8/9 (88.9%) | 8/9 (88.9%) |
| False-safe rate | 0/1 (0.0%) | 0/1 (0.0%) | 0/1 (0.0%) |
| First-tool accuracy | 10/10 (100.0%) | 9/10 (90.0%) | 10/10 (100.0%) |
| Sequence accuracy | 10/10 (100.0%) | 9/10 (90.0%) | 10/10 (100.0%) |
| Argument accuracy | 34/37 (91.9%) | 36/37 (97.3%) | 31/37 (83.8%) |
| Missing-input correctness | 1/1 (100.0%) | 0/1 (0.0%) | 0/1 (0.0%) |
| Parseable envelope | 10/10 (100.0%) | 10/10 (100.0%) | 9/10 (90.0%) |
| Median latency | 20.7 s | 34.2 s | 17.3 s |
| p95 latency | 33.6 s | 40.9 s | 42.6 s |

Figure 1. Exploratory ten-case local LLM screening. Percentages are model-selection evidence, not final reliability estimates.

Small-sample warning. Only nine cases had PASS/FAIL/ASK gold verdicts, and each model ran once. Qwen3.5's 9/9 result has a wide Wilson 95% interval of approximately 70.1% to 100%. The screening supports a practical model choice; it does not prove 100% reliability.

9. Candidate failure analysis and model decision

9.1 Qwen3.5 9B

Qwen3.5 was the only candidate to combine correct safety verdicts, correct route and sequence, correct missing-input handling, and fully parseable envelopes. Its three argument misses were omissions of values that happened to match tool defaults: sawtooth pulse shape and the default six-bottom/four-wall mount counts. These omissions did not change those pilot results, but they remain an instruction-following risk that the final benchmark stresses using non-default values.

9.2 Qwen3 14B

Qwen3 14B achieved the strongest argument score, but its missing-input failure was more serious. It invented `mass_kg=100`, called the selection tool, and returned PASS instead of ASK. It also added `series=ALL` in one case and broadened the catalogue scope beyond the intended default series. The larger model was slower and did not provide the safest controller behaviour.

9.3 Gemma 4 12B

Gemma routed tools correctly but omitted the explicit `best_isolation` objective, causing the deterministic selector to use its default objective and return a different part. It also failed the structured ASK envelope in the missing-mass case. Its fast median latency did not offset weaker argument fidelity and auditability.

Selected local LLM. Qwen3.5 9B, served through `cad-eval-qwen35-9b:8k`, was frozen for Systems B, C, and D. The decision is based on safety behaviour, tool route, missing-input handling, envelope compliance, latency, and GPU headroom rather than parameter count alone.

Part III - Local embedding and RAG selection

10. Role of the embedding model

The embedding model is active only when a RAG lookup is requested. It converts the query and reference passages into vectors. Cosine similarity then ranks the stored shock-mount chunks. The language model and embedding model therefore perform different jobs: Qwen3.5 decides and explains, while BGE-M3 retrieves candidate evidence.

Reference Markdown files in knowledge/shock_mount/
  -> local embedder converts each chunk to a vector
  -> vectors and stable metadata are saved in a JSON index
Question requiring a source
  -> query vector
  -> exact cosine similarity
  -> top three chunk IDs and excerpts
  -> Qwen3.5 explanation and citations

The choice to keep retrieval tool-triggered follows the system's separation of responsibilities. Numerical engineering questions should call deterministic tools. Reference, standards, assumption, workflow, or provenance questions may call `lookup_knowledge`. Making RAG always-on would add latency and context even when the answer should come entirely from the Python oracle.

11. Corpus and retrieval experiment

The initial local-embedding comparison used 38 focused Markdown chunks from `knowledge/shock_mount/` and 18 development relevance judgements: 16 in-scope queries and two out-of-scope/adversarial queries. Every candidate used the same texts, stable chunk IDs, metadata filter, exact cosine search, and ranking depths. No FAISS, reranker, sparse retrieval, query expansion, or LLM judge was used.

| Fixed item | Value |
| --- | --- |
| Corpus | Shock-mount Markdown chunks; one focused file per chunk |
| Ground truth | `evaluation/retrieval_qrels_shock_v1.jsonl` |
| Search | Exact dense cosine similarity in `JSONVectorStore` |
| Filter | `parent_topic=shock_mount` |
| Evaluation depths | Top 1, top 3, and top 5 |
| Primary operational depth | Top 3 because `lookup_knowledge` returns three passages |
| Hardware/backend | Same Windows laptop and local Ollama server |

12. Embedding model screening results

| Metric | BGE-M3 | Qwen3 Embedding 8B | MXBAI Embed Large v1 |
| --- | --- | --- | --- |
| Hit@1 | 81.25% | 75.00% | 62.50% |
| Recall@1 | 62.50% | 56.25% | 46.88% |
| Hit@3 | 100.00% | 93.75% | 100.00% |
| Recall@3 | 87.50% | 87.50% | 90.62% |
| Hit@5 | 100.00% | 100.00% | 100.00% |
| Recall@5 | 90.62% | 90.62% | 93.75% |
| MRR | 0.9062 | 0.8458 | 0.7812 |

Figure 2. Initial local dense-retrieval screening on 16 in-scope development queries.

BGE-M3 was selected because it achieved perfect Hit@3 and the strongest MRR, meaning the first relevant source appeared earlier on average. Qwen3 Embedding 8B was strong in physics and workflow questions but weaker on standards queries. MXBAI recovered relevant material by rank three but placed the first relevant passage lower on average.

Selected local embedder. BGE-M3 was frozen for System D. After the corpus was migrated from 38 to 34 focused chunks, BGE-M3 improved to Hit@1 = 87.5%, Hit@3 = 100%, Recall@3 = 87.5%, and MRR = 0.9271 on the same development qrels.

13. Frozen RAG configuration and pilot

| Setting | Frozen value |
| --- | --- |
| Embedding provider | Local Ollama `/api/embed` |
| Embedding model | `bge-m3` |
| Corpus | 34 files under `knowledge/shock_mount/` |
| Index | `artifacts/embedding_candidates/bge_m3.json` |
| Index SHA-256 | `00D972B51D26981FA07DAB64B27A121F6212A4ED781178A304C266FCD49CE528` |
| Search | Exact dense cosine similarity |
| Retrieval depth | Top 3 |
| Passage payload | First 1,400 characters per retrieved chunk |
| Stable citation ID | `shock_mount/<filename-stem>` |

A four-question System D development pilot verified the complete route. All four questions called `lookup_knowledge`, retrieved at least one required chunk, produced parseable information-only envelopes, and cited stable chunk IDs. The post-freeze pilot reported Hit@3 = 4/4, MRR = 1.0, and citation recall = 87.5%. The shortfall was retained as a measured model result rather than hidden as a pipeline error.

Part IV - Frozen 170-case evaluation

14. Research questions and hypotheses

The final experiment asks whether adding deterministic tools improves safety and correctness, and whether adding RAG supplies traceable references without damaging engineering-tool reliability.

| Hypothesis | Expected observation |
| --- | --- |
| H1 | System C has a lower false-safe rate than System B. |
| H2 | System C has higher PASS/FAIL/ASK verdict accuracy than System B. |
| H3 | System C correctly selects tools and fills arguments across selection, verification, conversion, and multi-tool cases. |
| H4 | System D retrieves and cites relevant frozen knowledge chunks on reference cases. |
| H5 | Adding RAG does not materially reduce D's engineering verdict, routing, argument, or envelope reliability compared with C. |
| H6 | System D has higher latency than C because local embedding and retrieval add work. |

15. B/C/D controlled ablation

| System | Configuration | Question answered |
| --- | --- | --- |
| B | Qwen3.5 9B only; no engineering tools; no RAG | How capable and safe is the raw local model without the deterministic system? |
| C | Same model plus deterministic shock/catalogue tools; no RAG | How much do validated Python tools improve safety and engineering correctness? |
| D | Same model plus the same tools and frozen BGE-M3 RAG | Does reference retrieval add traceability, and what latency or reliability cost does it introduce? |

This is a component-ablation design: B to C adds the engineering tool-enabled configuration; C to D adds the RAG-enabled configuration. The prompts cannot be literally identical because B must be told that tools are unavailable and D must receive retrieval/citation instructions. The defensible claim is therefore that the configurations differ by the capability and its necessary interface instructions, not that only one source-code line changed.

| Controlled variable | Frozen value |
| --- | --- |
| Base model and quantization | Qwen3.5 9B, Q4_K_M |
| Alias | `cad-eval-qwen35-9b:8k` |
| Temperature / seed | 0.0 / 42 |
| Context / maximum output | 8,192 / 2,048 tokens |
| Top-p | 0.95 |
| History | None; every benchmark case is independent |
| Repetitions | Three per case |
| Benchmark and oracle | Same frozen 170 cases and Python engineering oracle |
| Output contract | Same structured verdict envelope |

16. Final benchmark composition

| Category | Cases | Purpose |
| --- | --- | --- |
| Isolator selection | 35 | Choose a passing catalogue part under varied masses, mounts, shocks, objectives, and constraints. |
| Existing-part verification | 45 | Determine whether a stated part passes static, transmitted-acceleration, and travel gates. |
| Unit conversion | 10 | Preserve engineering meaning when mass is supplied in pounds. |
| Multi-tool workflow | 10 | Use catalogue data and analysis in the required order. |
| Missing input | 20 | Return ASK and identify required fields rather than inventing values. |
| Adversarial safety | 20 | Resist pressure to approve, assume mass, accept conflicting units, or process invalid values. |
| Reference questions | 30 | Measure retrieval and citation behaviour; information-only verdict is null. |
| Total | 170 | Three repeats under each of B, C, and D produce 1,530 raw runs. |

| Gold outcome | Cases | Scoring role |
| --- | --- | --- |
| PASS | 71 | Decision case |
| FAIL | 29 | Decision case and denominator for the primary false-safe metric |
| ASK | 40 | Decision case and missing-input safety evaluation |
| Information-only / null | 30 | RAG/reference case; excluded from verdict-accuracy denominator |

There are 140 PASS/FAIL/ASK decision cases. Forty-five numerical cases lie within the declared boundary band, including 40 of the 80 core selection and verification cases. Boundary cases matter because small unit, argument, or calculation differences can reverse the verdict near an acceptance threshold.

17. Dataset placeholder and repository location

DATASET PLACEHOLDER - RAW CASES INTENTIONALLY NOT PASTED. The full frozen benchmark contains 170 JSONL records and is maintained as a machine-readable repository artifact. Copying all records into Word would duplicate the source of truth and make the report difficult to maintain.

| Dataset or schema | Repository path | Use |
| --- | --- | --- |
| Final 170-case benchmark | `evaluation/benchmark/final_shock_draft_v0.jsonl` | Final B/C/D measurement |
| Final benchmark generator | `evaluation/benchmark/generate_final_shock_draft.py` | Reproducible case generation and oracle construction |
| Case schema | `evaluation/benchmark/schema.md` | Meaning of each JSONL field |
| Ten-case development set | `evaluation/benchmark/dev.jsonl` | Prompt, harness, and LLM screening development |
| RAG development pilot | `evaluation/benchmark/rag_dev_pilot.jsonl` | End-to-end RAG wiring |
| Retrieval qrels | `evaluation/retrieval_qrels_shock_v1.jsonl` | Embedding-model development comparison |

Recommended thesis treatment: include the benchmark composition table, schema summary, generation procedure, and three to five representative cases in an appendix. Keep the complete 170-case JSONL in the repository and cite its path and SHA-256 instead of reproducing every line.

PowerShell command to inspect the final dataset:
Get-Content evaluation\benchmark\final_shock_draft_v0.jsonl

Frozen benchmark SHA-256:
E27E129A9B6A2750E18D16C007D664B1427BBC715EF743EA0CE5C1ECB13A60ED

18. Execution and reproducibility procedure

Confirm the frozen Ollama alias and model digest.

Connect the laptop to AC power, retain the same performance mode, and close unrelated GPU-intensive applications.

Start Ollama and allow the runner to perform one unscored warm-up turn.

Run one system at a time; do not run B, C, and D concurrently.

Use the resume-safe JSONL output file so interrupted work skips completed case-repeat pairs.

Retain raw case ID, repeat, question, tool trace, tool results, retrieved chunk IDs, final text, parsed envelope, latency, retry state, and errors.

Score only with the frozen deterministic scorer. Do not tune the system based on final-case failures.

Aggregate the three repeats to a case-level majority verdict for paired statistical comparison.

The runner records 170 cases x 3 repeats = 510 records per system. Across B, C, and D the study contains 1,530 runs. Repeats measure consistency; they do not create 510 independent engineering cases per system.

19. Metrics and statistical analysis

19.1 Primary safety metric

False-safe rate. Number of gold FAIL cases predicted PASS, divided by the number of gold FAIL cases. This is the primary metric because an unsafe approval is more serious than a conservative rejection.

19.2 Secondary metrics

Verdict accuracy for PASS, FAIL, and ASK across 140 decision cases.

False-fail rate, ASK precision/recall/F1, and missing-field accuracy.

Tool decision, first-tool, ordered sequence, valid-schema, and no-tool abstention accuracy.

Per-argument accuracy with exact matching for enums/strings and declared tolerances for numeric conversions.

Parseable verdict-envelope rate; unparseable safety-critical outputs count as incorrect.

Retrieval Hit@3, Recall@3, MRR, citation precision/recall, and citation-validity rate for reference cases.

Three-repeat verdict consistency and descriptive latency p50/p95.

Hard failures such as loops, crashes, and tool execution errors.

19.3 Uncertainty and paired comparison

Every binomial rate should include a Wilson 95% confidence interval. Wilson intervals behave better than the simple normal approximation near zero or one and when the error count is small.

For B versus C and C versus D, each case's three repeats are converted to one majority outcome. Exact McNemar testing then uses only paired disagreements: cases where the first system is wrong and the second correct, and cases with the opposite pattern. This respects the paired benchmark design. Treating all 510 responses as independent would artificially inflate the sample size and confidence.

20. Why this evaluation design is credible

| Credibility feature | Contribution |
| --- | --- |
| Controlled ablation | The same selected model answers the same paired cases while tools and then RAG are added in a declared sequence. |
| Deterministic oracle | Gold verdicts, tools, arguments, and values come from Python rather than a subjective LLM judge. |
| Development/final separation | Prompt and retrieval tuning use development data; the frozen final set is measurement-only. |
| Artifact hashes | Model alias, prompt/tool manifest, benchmark, corpus/index, and source hashes make changes detectable. |
| Raw traces | Append-only JSONL preserves the question, calls, results, final answer, latency, and errors for audit. |
| Paired statistics | McNemar testing matches the fact that B, C, and D answer the same cases. |
| Uncertainty | Wilson intervals prevent small error counts from being reported as exact population truth. |
| Safety emphasis | False-safe rate, boundary cases, missing-input tests, and adversarial prompts reflect engineering risk. |

The method follows established ideas from agent/tool-use evaluation, fixed-query information retrieval, provenance evaluation, confidence intervals for proportions, and paired classification comparison. It is credible as a controlled project benchmark, but it is not a certification of universal or physical safety.

21. Frozen artifacts

| Artifact | Path or identifier | SHA-256 / value |
| --- | --- | --- |
| Selected model alias | `cad-eval-qwen35-9b:8k` | Digest `41877DCBD4E07E5C9C40F5AB7968BC651674D83801A171CFD6B0684D13701910` |
| Model/interface manifest | `evaluation/artifacts/model_interface_manifest.json` | `4946CFDF9386F8C350E76C7313726E30A849D72DA60610D2DF0B2996D0C22A6F` |
| Final benchmark | `evaluation/benchmark/final_shock_draft_v0.jsonl` | `E27E129A9B6A2750E18D16C007D664B1427BBC715EF743EA0CE5C1ECB13A60ED` |
| BGE-M3 index | `artifacts/embedding_candidates/bge_m3.json` | `00D972B51D26981FA07DAB64B27A121F6212A4ED781178A304C266FCD49CE528` |
| RAG corpus | `knowledge/shock_mount/` | 34 stable one-file chunks |

Change control. Any change to the model, quantization, serving settings, prompt, tool schema, engineering oracle, RAG index, benchmark, verdict parser, or scorer invalidates direct comparison with existing final runs. A changed system requires a new evaluation version and rerun of every compared configuration.

Part V - Results and progress record

22. Pre-final pilot validation

| Pilot | Result | Interpretation |
| --- | --- | --- |
| System B, 3 development cases | No tool calls; parseable envelopes | The model-only baseline is correctly isolated from deterministic tools. |
| System C v3, 3 development cases | 3/3 verdict, 3/3 first tool, 100% arguments, 3/3 envelope | The final engineering-verdict guard reliably ends with PASS, FAIL, or ASK. |
| System D v2, 4 RAG cases | 4/4 tool and envelope; Hit@3 4/4; MRR 1.0; citation recall 87.5% | The local BGE-M3 retrieval and citation path works end to end. |

23. System B final results

Completion status. System B is complete: 170 cases x 3 repeats = 510 records. The scorer found zero hard execution failures.

| Metric | System B result | Wilson 95% interval / note |
| --- | --- | --- |
| Verdict accuracy | 162/420 (38.6%) | 34.0% to 43.3%; per-run decision responses |
| False-safe rate | 12/87 (13.8%) | 8.1% to 22.6%; primary safety metric |
| False-fail rate | 3/213 (1.4%) | 0.5% to 4.1% |
| Parseable envelope | 501/510 (98.2%) | 96.7% to 99.1% |
| Missing-input accuracy | 93/120 (77.5%) | 69.2% to 84.1% |
| Three-repeat consistency | 129/140 (92.1%) | 86.5% to 95.6%; decision cases |
| Hard failure | 0/510 (0.0%) | Upper Wilson bound approximately 0.75% |
| Latency | p50 14.1 s; p95 66.0 s | Descriptive; systems are run sequentially |

The current scored records also produce the following case-level majority outcomes. The majority verdict is identical across at least two of the three repeats for every decision case, but 11 cases are inconsistent across all three repetitions and are reflected in the 92.1% consistency score.

| Gold | Majority PASS | Majority FAIL | Majority ASK | Majority null | Correct |
| --- | --- | --- | --- | --- | --- |
| PASS (71) | 21 | 1 | 42 | 7 | 21/71 |
| FAIL (29) | 4 | 0 | 21 | 4 | 0/29 |
| ASK (40) | 7 | 0 | 33 | 0 | 33/40 |
| Total (140) | 32 | 1 | 96 | 11 | 54/140 (38.6%) |

At case level, the raw model produced 4 false-safe outcomes among 29 gold FAIL cases, again 13.8%. It often responded conservatively with ASK or null rather than making the required engineering decision. This is exactly the baseline behaviour that System C is intended to improve by supplying the validated tools.

Important denominator note. System B's reported 100% first-tool and sequence scores mean it correctly made no tool calls under a no-tool configuration. They measure abstention, not engineering tool-call capability. Tool-required routing and argument metrics become meaningful in Systems C and D.

24. Current C/D placeholders

| System | Current raw progress at 15 July 2026 | Result status |
| --- | --- | --- |
| B | 510/510 records | Complete and scored |
| C | 66/510 records in `evaluation/runs/C_shock_final_v1.jsonl` | Run in progress; do not report final percentages |
| D | No `D_shock_final_v1.jsonl` file yet | Not started |

SYSTEM C RESULT PLACEHOLDER. Insert final C metrics, confidence intervals, majority confusion matrix, tool/argument breakdown, latency, and B-versus-C McNemar table after 510 records are complete and scored.

SYSTEM D RESULT PLACEHOLDER. Insert final D engineering metrics, retrieval/citation metrics, latency, failure analysis, and C-versus-D McNemar table after 510 records are complete and scored.

25. Challenges encountered and engineering responses

| Challenge | Observed effect | Response and lesson |
| --- | --- | --- |
| Local provider was not originally implemented | The app recognised only cloud provider branches. | Added a local Ollama configuration and OpenAI-compatible base URL while retaining cloud code as a separate path. |
| Ollama model disappeared from `ollama ps` | It appeared that the app might not be using the local model. | Confirmed GPU residency immediately after a request; documented that keep-alive unloading is normal. |
| Qwen3 thinking and default context | Responses could be slow and context was initially 4K. | Created immutable 8K evaluation aliases with temperature, seed, output cap, and thinking controls. |
| Missing ASK envelope | Correct prose could not be scored as ASK. | Added a literal evaluation-only ASK example before final benchmarking. |
| Larger model invented mass | Qwen3 14B called a tool using an unstated 100 kg mass. | Made missing-input behaviour a primary model-selection criterion rather than relying on parameter count. |
| Explicit values omitted when equal to defaults | Qwen3.5 could obtain the right result while weakening traceability. | Retained per-argument scoring and designed final cases with non-default values. |
| Hosted embeddings conflicted with the offline goal | RAG still depended on an external API. | Screened three local embedders and froze BGE-M3. |
| Legacy/overlapping corpus pages | Retrieval rankings were diluted and scope boundaries were unclear. | Migrated the corpus from 38 to 34 focused chunks and reran retrieval evaluation. |
| RAG payload threatened the 8K context | Three full pages plus tool schemas could crowd the prompt. | Capped each retrieved passage at 1,400 characters while retaining full text in the index. |
| Engineering answers sometimes ended in null | C produced information-only envelopes after tool use. | Added an evaluation guard requiring PASS/FAIL/ASK after engineering tools; C v3 passed 3/3. |
| Formal throughput/VRAM capture was not implemented before B | Token counts are null and no comparable B peak-VRAM trace exists. | Do not claim these as final comparative metrics. Treat latency and existing Ollama observations as descriptive only. |

26. Remaining work

Complete the remaining System C records in the existing resume-safe output file.

Score System C and inspect category, boundary, missing-input, tool-route, sequence, argument, envelope, consistency, and latency results.

Run all 510 System D records using the frozen BGE-M3 index and unchanged benchmark.

Score System D engineering, retrieval, citation, consistency, and latency metrics.

Implement or run case-level majority aggregation, PASS/FAIL/ASK confusion matrices, ASK precision/recall/F1, subgroup tables, and exact McNemar comparisons.

Classify representative failures without tuning or rerunning the same final benchmark.

Update this masterbook with completed C/D tables, paired statistics, discussion, and final conclusions.

Create the outstanding repository freeze tag only after the intended release gates are honestly satisfied.

27. Threats to validity and claim boundaries

| Threat | Meaning for the report |
| --- | --- |
| Single selected model | Final B/C/D findings apply to Qwen3.5 9B Q4_K_M, not all local LLMs. |
| Synthetic/oracle-backed cases | The benchmark is controlled and auditable but may not match the full distribution of real engineer queries. |
| One domain | The final study is shock-mount-specific; tiedown and mobility are outside the frozen evaluation scope. |
| One hardware/backend stack | Latency and resource behaviour are specific to this Windows/Ollama/RTX 5080 laptop setup. |
| Oracle assumptions | Agent correctness inherits the scope and limitations of the frozen catalogue and physics implementation. |
| No independent explanation-quality judge | RAG can be evaluated for retrieval and citation support, but not every nuance of prose correctness. |
| Sequential execution | Background load and thermal state can affect latency; latency remains descriptive. |
| No comparable token/peak-VRAM capture | These measures must be omitted from confirmatory B/C/D claims unless B is rerun under a declared supplementary protocol. |

Appropriate conclusion boundary. The study can establish controlled agreement with a frozen engineering oracle and quantify the contribution of tools and RAG. It cannot claim certified shock safety, universal LLM reliability, or replacement of physical testing and engineering review.

Part VI - Learning and evidence map

28. Recommended learning sequence

The most efficient learning order is to understand the model, then the tool loop, then retrieval. The original Transformer paper becomes easier to read after these practical foundations.

1. Transformer foundations: 3Blue1Brown: Transformers, the tech behind LLMs; then Attention in transformers, step by step. Open resource

2. Tool calling: LangChain Tools and Agents documentation. Compare the examples with the project's shock tools in `agent.py`. Open resource

3. Agent loop: LangChain Agents documentation for sequential model-to-tool execution and result return. Open resource

4. RAG: Hugging Face Agentic RAG course and Transformers RAG overview. Open resource

Study flow:
transformer/token fundamentals
  -> LangChain tool schemas and agent loop
  -> embeddings and RAG retrieval
  -> trace one real question through agent.py, the Python tool, and the verdict envelope

29. Repository evidence map

| Evidence | Repository location |
| --- | --- |
| LLM screening narrative | `evaluation/SCREENING_PROGRESS_2026-07-13.md` |
| Embedding screening narrative | `evaluation/EMBEDDING_SCREENING_PROGRESS_2026-07-14.md` |
| Evaluation protocol | `evaluation/PROTOCOL.md` |
| Freeze record | `evaluation/FREEZE.md` |
| Environment record | `evaluation/ENVIRONMENT.md` |
| System configurations | `evaluation/harness/systems.py` |
| Runner | `evaluation/harness/runner.py` |
| Verdict parser | `evaluation/harness/verdict.py` |
| Scoring | `evaluation/scoring/metrics.py` and `evaluation/scoring/scorer.py` |
| LLM pilot raw/scored outputs | `evaluation/runs/*dev_pilot_v2.jsonl` and `evaluation/results/*dev_pilot_v2_scored.json` |
| Embedding results | `evaluation/results/*retrieval*.json` |
| System B final raw/scored | `evaluation/runs/B_shock_final_v1.jsonl` and `evaluation/results/B_shock_final_v1_scored.json` |
| Shock RAG corpus | `knowledge/shock_mount/` |
| Frozen BGE-M3 index | `artifacts/embedding_candidates/bge_m3.json` |
| Model/interface manifest | `evaluation/artifacts/model_interface_manifest.json` |

30. Suggested slide extraction map

| Slide | Content to reuse |
| --- | --- |
| 1. Problem | Local, private engineering assistant; LLM as controller, Python as oracle. |
| 2. Architecture | Question -> Qwen -> deterministic tool or RAG -> answer/envelope. |
| 3. Hardware decision | 16 GB VRAM, Q4_K_M, 8K context, local Ollama. |
| 4. LLM screening | Three candidates, Figure 1, selected Qwen3.5 9B. |
| 5. Embedding screening | Three candidates, Figure 2, selected BGE-M3. |
| 6. Experimental design | B/C/D ablation table and controlled variables. |
| 7. Benchmark | 170-case composition, three repeats, safety/boundary coverage. |
| 8. Metrics | False-safe primary; verdict, tool, argument, RAG, consistency, latency secondary. |
| 9. Current result | System B table and majority confusion matrix. |
| 10. Next/result conclusion | C/D placeholders now; final paired comparison after completion. |

Below is a cleaner complete workflow. The technical term for your LangChain mechanism is agentic tool calling, also called function calling or tool-augmented LLM orchestration.

Complete Project Workflow Plan

## Project Objective

Develop and evaluate a secure, locally deployable engineering assistant for shock-mount selection. The platform combines:

- A transparent engineering user interface.

- A local large language model.

- Deterministic Python engineering tools.

- A local retrieval-augmented generation system.

- A controlled evaluation framework.

The language model interprets and coordinates the workflow, while validated Python functions remain responsible for calculations and engineering verdicts.

## Stage 1: Define the Engineering Problem

Identify weaknesses in the existing workflow:

- Manual transfer of information between CAD, spreadsheets, catalogues and reports.

- Incorrect values or units.

- Missing engineering inputs.

- Loss of source provenance.

- Difficulty reconstructing how a safety verdict was reached.

The project therefore addresses not only calculation speed, but also safety, accuracy, traceability and workflow consistency.

## Stage 2: Design the Platform Architecture

Separate the platform into two main parts:

1. **Deterministic engineering interface**

Allows users to enter or inspect inputs, run calculations and view intermediate results directly. This provides transparency and user confidence.

2. **AI engineering assistant**

Uses a local LLM for natural-language understanding, agentic tool calling and result explanation.

Responsibilities are separated as follows:

- The LLM interprets the question and selects an appropriate tool.

- Python tools perform catalogue lookup, unit conversion and shock-isolation calculations.

- RAG retrieves supporting engineering references.

- The Python tools, not the LLM, determine calculation results and safety verdicts.

## Stage 3: Establish Local Deployment Constraints

Determine what can run on the available enterprise hardware.

This includes:

- GPU VRAM and system memory.

- Model weight size and quantisation.

- Context-window and KV-cache requirements.

- Inference latency.

- Data-security and offline-deployment requirements.

Only models that fit the available hardware are considered for screening.

## Stage 4: Screen and Select the Local LLM

Compare candidate local models using the same ten-case development benchmark:

- Qwen3.5 9B.

- Qwen3 14B.

- Gemma 4 12B.

Keep the following conditions constant:

- Quantisation.

- Temperature and seed.

- Context window.

- Output-token limit.

- System prompt.

- Tool descriptions and argument schemas.

- Test questions.

- RAG disabled.

Evaluate:

- Verdict accuracy.

- Tool selection.

- Argument extraction.

- Missing-input behaviour.

- Output-envelope compliance.

- Latency.

**Selected model:** Qwen3.5 9B.

It provided the strongest balance of tool-use accuracy, safety and auditability, particularly because it correctly requested missing mass rather than inventing a value.

## Stage 5: Build and Evaluate Local RAG

Create a shock-mount knowledge corpus containing:

- MIL-STD-810H shock guidance.

- Manufacturer catalogue information.

- Selection and verification workflows.

- Engineering assumptions and limitations.

- Project-specific requirements.

Convert the source passages into embeddings and compare local embedding models:

- BGE-M3.

- Qwen3 Embedding 8B.

- MXBAI Embed Large.

Evaluate retrieval using:

- Hit@1.

- Hit@3.

- Recall@k.

- Mean reciprocal rank.

- Out-of-scope retrieval behaviour.

**Selected embedding model:** BGE-M3.

Freeze the selected corpus, embedding model, chunk identifiers, retrieval settings and top-k value.

## Stage 6: Freeze the Experimental Configuration

Before the main evaluation, freeze:

- Qwen3.5 9B model and quantisation.

- BGE-M3 embedding model.

- Context and output-token limits.

- Temperature and seed.

- Prompts and tool schemas.

- Shock-mount knowledge corpus.

- Benchmark cases.

- Verdict-output format.

- Software and hardware environment.

Record hashes and configuration manifests so the experiment can be reproduced.

## Stage 7: Conduct the Main B/C/D Experiment

Evaluate three controlled configurations:

- **System B:** local LLM only.

- **System C:** same LLM plus deterministic Python tools.

- **System D:** same LLM, tools and local RAG.

This creates two controlled comparisons:

- **B to C:** measures the contribution of deterministic tools.

- **C to D:** measures the additional contribution of RAG.

Use the same frozen model, settings and benchmark for all systems.

## Stage 8: Run the 170-Case Benchmark

Use 170 independent shock-mount cases covering:

- Isolator selection.

- Engineering verification.

- Unit conversion.

- Multi-tool workflows.

- Missing information.

- Invalid or conflicting inputs.

- Adversarial safety requests.

- Reference and standards questions.

Run every case three times under B, C and D:

- 510 runs per system.

- 1,530 runs in total.

The repeats measure consistency. Statistical analysis treats each benchmark case, rather than each repeated answer, as the main experimental unit.

## Stage 9: Score and Analyse the Results

Measure four main areas:

### Engineering safety

- Verdict accuracy.

- False-safe rate.

- False-fail rate.

- Boundary-case performance.

- Correct `ASK` behaviour.

### Tool-use quality

- Correct first tool.

- Correct tool sequence.

- Argument accuracy.

- Schema validity.

- Unnecessary or missing tool calls.

### RAG traceability

- Retrieval Hit@3.

- Recall and mean reciprocal rank.

- Citation precision and recall.

- Whether cited passages were actually retrieved.

### Operational performance

- Response consistency.

- Median and 95th-percentile latency.

- Hard failures.

- Token usage and VRAM consumption.

Use Wilson confidence intervals and exact McNemar tests for paired system comparisons.

## Stage 10: Analyse Failures

Keep the completed B/C/D results unchanged as baseline version 1.

Classify failures into:

- Tool or Python implementation errors.

- Incorrect tool selection.

- Incorrect argument extraction.

- Missing-input handling errors.

- Verdict-envelope errors.

- Retrieval failures.

- Citation failures.

- Genuine model-reasoning failures.

This identifies which component should be improved instead of assuming every failure requires fine-tuning.

## Stage 11: Develop the Improved System

Create a separate development dataset based on the identified failure categories. Do not train or tune directly on the original 170 final cases.

Apply improvements in this order:

1. Correct deterministic tool and data problems.

2. Improve tool descriptions or argument schemas.

3. Improve missing-input and output-format guards.

4. Improve RAG documents or retrieval settings.

5. Use LoRA/SFT only for remaining model-behaviour failures.

Freeze the improved model, adapter, prompts, tools and RAG configuration.

## Stage 12: Final E/F Evaluation

Evaluate the adapted system on a new untouched holdout benchmark:

- **System E:** adapted LLM plus deterministic tools.

- **System F:** adapted LLM plus deterministic tools and RAG.

Compare E/F against the B/C/D baseline to determine:

- Whether adaptation improves safety and reliability.

- Which failure categories improve.

- Whether improvements introduce regressions.

- Whether the additional training complexity is justified.

## Stage 13: Complete the Thesis and Demonstration

Produce:

- The final Streamlit engineering assistant.

- Reproducible configuration and benchmark records.

- Model and embedding selection results.

- B–F comparison tables.

- Statistical findings.

- Failure taxonomy.

- System limitations.

- Recommendations for secure enterprise deployment.

- Final report, presentation and live demonstration.

## Complete Research Flow

```text

Define engineering problem

→ design local agent architecture

→ establish hardware constraints

→ screen and select local LLM

→ build and select local RAG

→ freeze configuration

→ run B/C/D controlled experiment

→ analyse safety, accuracy and traceability

→ classify failures

→ improve tools, RAG and model behaviour

→ run E/F on an untouched holdout

→ complete thesis and demonstration

```

## Central Research Contribution

The project is not simply a local chatbot or shock-mount calculator. Its main contribution is a reproducible framework for determining:

- Whether deterministic engineering tools improve a local LLM’s safety and correctness.

- Whether local RAG improves traceability without reducing reliability.

- How evaluation evidence can identify whether a failure should be addressed through code, prompting, retrieval or model adaptation.

References

3Blue1Brown. Neural networks and transformer visual lessons. https://www.3blue1brown.com/topics/neural-networks

LangChain. Tools documentation. https://docs.langchain.com/oss/python/langchain/tools

LangChain. Agents documentation. https://docs.langchain.com/oss/python/langchain/agents

Hugging Face. Agentic Retrieval-Augmented Generation course. https://huggingface.co/learn/agents-course/en/unit3/agentic-rag/agentic-rag

Hugging Face Transformers. Retrieval-Augmented Generation overview. https://huggingface.co/docs/transformers/v4.51.3/model_doc/rag

Liu et al. AgentBench: Evaluating LLMs as Agents. ICLR 2024. https://proceedings.iclr.cc/paper_files/paper/2024/hash/e9df36b21ff4ee211a8b71ee8b7e9f57-Abstract-Conference.html

Qin et al. ToolLLM: Facilitating Large Language Models to Master 16000+ Real-world APIs. ICLR 2024. https://openreview.net/forum?id=dHng2O0Jjr

NIST. TREC evaluation measures. https://trec.nist.gov/pubs/trec16/appendices/measures.pdf

Thakur et al. BEIR: A Heterogeneous Benchmark for Zero-shot Evaluation of Information Retrieval Models. https://openreview.net/forum?id=wCu6T5xFjeJ

Petroni et al. KILT: a Benchmark for Knowledge Intensive Language Tasks. NAACL 2021. https://aclanthology.org/2021.naacl-main.200/

Brown, Cai and DasGupta. Interval Estimation for a Binomial Proportion. Statistical Science, 2001. https://doi.org/10.1214/ss/1009213286

McNemar. Note on the sampling error of the difference between correlated proportions or percentages. Psychometrika, 1947. https://doi.org/10.1007/BF02295996

Appendix A - Reusable terminology

| Term | Project-specific meaning |
| --- | --- |
| Local LLM | A language model whose inference runs on the laptop through Ollama rather than a hosted chat endpoint. |
| Quantization | Lower-precision storage of model weights to reduce memory and permit local inference. |
| Context window | Maximum active tokens available to the model for instructions, tool schemas, question, tool results, and answer. |
| KV cache | Runtime memory storing attention keys and values for tokens already processed in the active context. |
| Tool calling | The model emits a structured function name and arguments; LangChain executes the Python function and returns its result. |
| RAG | Retrieval of external text passages that are inserted into the model's context to support an answer. |
| Embedding | A vector representation used to compare semantic similarity between a query and knowledge chunks. |
| Oracle | The frozen deterministic catalogue/physics code used to create gold benchmark outcomes. |
| Ablation | A controlled comparison that adds or removes a system component to measure its associated contribution. |
| False-safe | A gold FAIL case that the evaluated system predicts as PASS. |
| Wilson interval | A confidence interval for a binomial proportion with better behaviour than the simple normal interval. |
| McNemar test | A paired test based on cases where two systems disagree in correctness. |

Appendix B - Report-ready method paragraph

A controlled component-ablation experiment was used to evaluate the locally hosted agent. A single frozen Qwen3.5 9B Q4_K_M model answered the same 170 shock-mount benchmark cases under three configurations: model-only (B), model plus deterministic engineering tools (C), and model plus tools and frozen BGE-M3 retrieval (D). Each case was repeated three times at temperature zero using an 8,192-token context and a fixed JSON verdict envelope. Numerical ground truth, expected tools, arguments, and safety verdicts were generated by the frozen Python engineering oracle, allowing deterministic scoring without an LLM judge. The primary metric was false-safe rate, supported by verdict accuracy, ASK handling, tool selection, argument accuracy, retrieval and citation metrics, consistency, latency, Wilson 95% confidence intervals, and exact McNemar tests on paired majority case outcomes.

Appendix C - Report-ready model-selection paragraph

Three Q4_K_M local instruction models were screened under identical serving and tool-use conditions: Qwen3.5 9B, Qwen3 14B, and Gemma 4 12B. Qwen3.5 9B was selected because it was the only candidate to achieve correct verdicts across all nine scoreable safety cases, correct first-tool routing across all ten cases, correct missing-input ASK behaviour, and fully parseable output envelopes. Qwen3 14B extracted explicit arguments accurately but invented a missing mass and broadened catalogue scope, while Gemma 4 12B omitted a stated optimization objective and failed one ASK envelope. Because the pilot contained only ten cases with one repeat, it is treated as exploratory model-selection evidence rather than final reliability evidence.

Appendix D - Report-ready embedding-selection paragraph

Three locally served dense embedding models were evaluated using the same shock-mount corpus, stable chunk identifiers, exact cosine similarity search, and development relevance judgements. BGE-M3 achieved Hit@3 of 100% and the strongest mean reciprocal rank of 0.9062, compared with 0.8458 for Qwen3 Embedding 8B and 0.7812 for Mixedbread MXBAI Embed Large v1. BGE-M3 was therefore selected and the index was rebuilt after corpus migration. The frozen 34-chunk index subsequently achieved Hit@1 of 87.5%, Hit@3 of 100%, Recall@3 of 87.5%, and MRR of 0.9271 on the development qrels. These results select the retrieval component; final RAG effectiveness is measured separately on the disjoint reference cases in System D.

End of progress edition. Replace the System C and D placeholders only after their frozen runs and deterministic scoring are complete. Preserve this edition as a dated project record.